GENERAL

Location/name of enclosed space :

Reason for entry  :  .

This permit is valid      From:                 hrs	Date   ...............

To:                     hrs	Date  ...............

(See Note 1)

SECTION 1 – PRE-ENTRY PREPARATION

(To be checked by the master or nominated responsible person)

| - Has the space been thoroughly ventilated by mechanical means? - Has the space been segregated by blanking off or isolating all connecting pipelines or valves and electrical Power/equipment? | Yes / No |  |
| --- | --- | --- |
| - Has the space been cleaned where necessary? |  |  |
| - Has the space been tested and found safe for entry? (See note 2) |  | . |
| - Pre-entry atmosphere test readings: |  |  |
| - oxygen %vol (21%)* | By: |  |

- hydrocarbon	% LFL (less than 1%)

- toxic gases ............. ppm (less than 50% OEL of the specific gas) Time:

(See note 3)

- Have arrangements been made for frequent atmosphere checks to

be made while the space is occupied and after work breaks?

- Have arrangements been made for the space to be continuously

ventilated throughout the period of occupation and during work breaks?

- Are access and illumination adequate?

- Is rescue and resuscitation equipment available for immediate use by

the entrance to the space?

- Has an attendant been designated to be in constant attendance at the

entrance to the space?

- Has the officer of the watch (bridge, engine-room, cargo control room)

been advised of the planned entry?

- Has a system of communication between all parties been tested  and

emergency signals agreed?

- Are emergency and evacuation procedures established and

understood by all personnel involved with the enclosed space entry

- Is all equipment used in good working condition and inspected prior

to entry?

- Are personnel properly clothed and equipped?

SECTION 2 – PRE-ENTRY CHECKS

(To be checked by each person entering the space)

Yes/  No

  - I have received instructions or permission from the master or

nominated responsible person to enter the enclosed space

  - Section 1 of this permit has been satisfactorily completed by the

master or nominated responsible person"

  - I have agreed and understand the communication procedures

  - I have agreed upon a reporting interval of .............. minutes

  - Emergency and evacuation procedures have been agreed and are

understood

  - I am aware that the space must be vacated immediately in the event of ventilation failure or if atmosphere tests show a change from agreed safe criteria

SECTION 3 –

BREATHING APPARATUS AND OTHER EQUIPMENT

(To be checked jointly by the master or nominated responsible  person and the person who is to enter the space)

Yes/  No

- Those entering the space are familiar with any breathing

apparatus to be used

- The breathing apparatus has been tested as follows:

- gauge and capacity of air supply

- low pressure audible alarm if fitted

- face mask – under positive pressure and not leaking

- The means of communication and emergency signals Tested/ agreed.

- All personnel entering the space have been provided with rescue

harnesses and, where practicable, lifelines

Signed upon completion of sections 1, 2 and 3 by

Master or nominated responsible person :                                  Date :                  Time:

Attendant :                                                                                Date :                  Time:

Person entering the space :                                                          Date :                   Time :

SECTION 4 – PERSONNEL ENTRY

(To be completed by the responsible person supervising entry)

Names  :

Time in :                                Time out:

SECTION 5 – COMPLETION OF JOB

(To be completed by the responsible person supervising entry)

Job completed	                    Date:         	Time  :

- Space secured against entry - 	                               Date:	         Time :

- The officer of the watch has been duly informed -	   Date:	         Time  :

Signed upon completion of sections 4 and 5 by:

Responsible person supervising entry –                            Date :                Time :

# Notes:

- The permit should contain a clear indication as to its maximum period of validity.

- In order to obtain a representative cross-section of the space's atmosphere, samples should be taken from several levels and through as many openings as possible. Ventilation should be stopped for about 10 minutes before the pre-entry atmosphere tests are taken.

- Tests for specific toxic contaminants, such as benzene or hydrogen sulphide, should be undertaken depending on the nature of the previous contents of the space.

THIS PERMIT IS RENDERED INVALID SHOULD VENTILATION OF THE SPACE STOP                     OR IF ANY OF THE CONDITIONS NOTED IN THE CHECKLIST CHANGE