- WEEKLY ROUTINE :

| No. | Item | Checks & Maintenance | Done On | Next Due |
| --- | --- | --- | --- | --- |
| 1 | Lifeboat Davits and Winches | Davits & Winches to lubricate by swinging out free of the limit switch (weather permitting)Wires and pulleys to be greased. |  |  |
|  |  | Davit limit switches to be checked for correct operation. |  |  |
|  |  | Davit air/electric motors to check for operation. |  |  |
| 2 | Day Light Signaling Lamps | Check lamps are operating correctly. Spare bulbs available. |  |  |
|  |  | The source of electrical power separate from the ship’s main source of electrical power is operating correctly. |  |  |
|  |  | If operated by rechargeable batteries check and confirm batteries always kept on trickle charge + charger operating satisfactorily. |  |  |
| 3 | ##### Emergency Headquarters and Supplementary Equipment Stations including Breathing Apparatus | Muster all equipments and check that it is in good working order. Rectify defects |  |  |
|  |  | Wash facemasks of breathing apparatus with weak soap solution (not detergent),Rinse thoroughly in fresh water and allow to dry under normal condition away from heat. Leave straps fully extended. |  |  |
| 4 | General Emergency Alarm System | The alarm system should be tested using any alarm button. All alarm bells should be checked as operating. |  |  |
| 5 | ##### Fixed Fire Fighting Arrangements for P/Rm. E/ Rm. Stores & Main Deck (CO2/ Foam) | Alarms and trips associated with system to try out and confirm good working order. |  |  |
|  |  | Visual inspection of system for corrosion, clamping, operation and readiness |  |  |
| 6 | Fire Detection System | Test approximately 10 % of the detector heads for correct operation, replace any defective heads and return for servicing. |  |  |
|  |  | The Electrician should keep a record book of detector heads tested showing a rotational system covering all heads at least once every 3 months. |  |  |
| 7 | ##### Lifeboat, Liferaft and Embarkation Dk. Emergency Lights | Lights to be tested and defects made good. |  |  |
| 8 | ##### Fixed Foam Extinguishing System | Re-circulate foam liquid for approximately 15 minutes. |  |  |
| 9 | Emergency Fire Pump | Check lubricating oil level.Start and run pump for at least five minutes. |  |  |
|  |  | Check line pressure with at least two (2) hydrants open. |  |  |
|  |  | Check fuel tank – Full. |  |  |
|  |  | Start by local and remote starts on alternate weeks. |  |  |
| 10 | Lifeboat Engines. | Lubricating oil and cooling water-check level and adjust if necessary. |  |  |
|  |  | Engines to be run, throttle and gear levers operated in ahd/ast direction. |  |  |
|  |  | Fuel tanks to be checked – Full. |  |  |
| 11 | E/ Room Fire Flaps and Dampers | All fire flaps and dampers to be tested and lubricated as necessary. |  |  |
|  |  | Check in good condition and closing correctly (fully). |  |  |
|  |  | Check marked correctly. |  |  |
| 12 | Galley Exhaust Fan Filters | Remove filters and clean using hot water and strong soap solution. Rinse with fresh water and dry. Wipe clean around filter housing and trunking. |  |  |
|  |  | Clean drains and remove excess fat/ oil from fan hood gutters. Replace filters. |  |  |

- MONTHLY ROUTINE :

| No. | Item | Checks & Maintenance | Done On | Next Due |
| --- | --- | --- | --- | --- |
| 1 | Pilot Ladders and Hoists | Inspect condition of side ropes, manropes, steps and battens. Renew as necessary. |  |  |
|  |  | Check ladders comply with SOLAS 1974, Ch. 5, Reg. 17. |  |  |
| 2 | # Galley Exhaust Grills and Trunks | Check all galley exhaust grills and trunking for accumulated fat and grease. Clean as required. |  |  |
| 3 | Multi-Gas Detectors | Check instrument fully charged. |  |  |
|  |  | Check pump suction operation. |  |  |
|  |  | Check sufficient ‘in date’ absorption tubes carried (H2S/ CO/ CO2). |  |  |
| 4 | ##### E.P.I.R.B. and S.A.R.T. | Check condition of float free and survival craft beacons, casings and mountings. |  |  |
|  |  | Check battery renewal date. |  |  |
|  |  | Test E.P.I.R.B. operation in accordance with manufacturer’s instructions. |  |  |
| 5 | ##### VHF Hand Sets (Walkie Talkies) | Check condition of 3 sets survival craft 2-way radios. |  |  |
|  |  | Check operation on Channel 16 and at least 2 other channels. |  |  |
| 6 | Safety & Rescue Harnesses, Lines and Lifelines | Rescue and Safety Harnesses to be inspected for damage to webbing or buckles. Damaged harnesses to be condemned, mutilated and disposed of, and then replaced with new harnesses. |  |  |
|  |  | Rescue and Safety Harnesses to be inspected for damage to webbing or buckles. Damaged harnesses to be condemned, mutilated and disposed of, and then replaced with new harnesses. |  |  |
|  |  | Inspect rescue lines (to be of at least 20mm dia. Cordage) and safety lines for damage or contamination by oil or paint. Replace any suspect lines immediately. |  |  |
|  |  | Check condition and lubricate all blocks, sheaves, cleats and shackles which are part of the rescue system. |  |  |
| 7 | Fire Extinguisher | All fire extinguishers are to be entered on the Fire Extinguishers Service and Record Sheets. |  |  |
|  | NOTE : The maintenance of Fire Extinguishers is critical to the safety of the vessel. They are the first line of defense against fire and correct operation can be the difference between the saving or loss of life and vessel. | During the current month, extinguishers indicated on the service chart should be maintained as follows: - a)DCP extinguishers to have caps removed, inverted and powder shaken loose. Clear discharge hoses of loose powder, check nozzle valves for correct operation, check pressure relief holes in caps are clear. b) Check weigh powder charge and CO2 discharge cartridges. c)Water filled extinguishers should be refilled, hoses checked for blockage and deterioration. Cap threads should be lightly greased with Vaseline. d)On an annual basis foam extinguishers should be operated checked for excessive corrosion, cleaned are refilled |  |  |
| 8 | Lifejackets and Survival Suits. | Condition of lifejackets and whether properly marked. |  |  |
|  |  | Lifejacket light and battery, whistles secured to lifejackets by lanyard are in good condition and operating correctly. |  |  |
|  |  | Infant and Oversize lifejackets as per requirement. |  |  |
|  |  | Reflective tape in good condition. (‘M’ Notice 1444).Donning instructions posted. |  |  |
| 9 | Fixed Gas Extinguishing Systems | Test alarms and releases on CO2 systems. Blow through lines with air. |  |  |
|  |  | Grease release fittings. Ensure release instructions are clear. |  |  |
|  |  | Check operation of padlocks on release boxes and CO2 bottle room. |  |  |
|  |  | Check that correct keys are stowed in “Break Glass to Release” boxes close to associated locks. |  |  |
| 10 | Emergency/Fire Alarms, Bells, Sirens and Switches | All alarm switches to be operated and bells and sirens checked for correct operation. (The Location of all bells, alarms and manual actuation points is indicated on the vessel’s Fire Fighting Plan). |  |  |

| No. | Item | Checks & Maintenance | Done On | Next Due |
| --- | --- | --- | --- | --- |
| 11 | Fire Flaps and Dampers | Operate all fire flaps and dampers. Lubricate working parts and grease wires. |  |  |
|  | NOTE : Handles to be painted in distinctive colors and the “OPEN” and “SHUT” position of handles to be marked prominently. Space served by a fire flap is to be indicated either adjacent to or on the handle of flap. | Check condition of rubber gaskets and confirm positive sealing |  |  |

- QUATERLY ROUTINE :

| No. | Item | Checks & Maintenance | Done On | Next Due |
| --- | --- | --- | --- | --- |
| 1 | Stretchers and associated fittings | Check 2 lifting ropes of at least 20mm in good condition and free of wear and tear. |  |  |
|  |  | Check stretcher lifting straps/ ropes in good order. Check the stretcher condition. |  |  |
|  |  | Check steadying line in good order. |  |  |
|  |  | Check pulleys and shackle of the davit for free movements and free of wear and tear. |  |  |
|  |  | Lubricate all the moving parts of the lifting davit . |  |  |
| 2 | Lifeboats/Rescue boat | Lifeboats/Rescue to be lowered into the water (vessel at anchor), fall release mechanism test and lubricate, check correct operation of engine and steering gear. |  |  |
|  |  | # Renew fresh water. |  |  |
| 3 | Isolating Valves | Operate all valves through full limits. Lightly grease working parts. Seized valves to be repaired. |  |  |
|  |  | If required repaint valve wheels: - Fire: RED, Foam: YELLOW |  |  |
| 4 | Foam Monitors | Grease all working parts and operate monitors through full limits of location and elevation. |  |  |
| 5 | Fire House Boxes/Racks | Check that all boxes and racks contain equipment, all washers in place and nozzles operating. |  |  |
| 6 | Fire Extinguishers | Check that all extinguishers are in position listed in Establishment Inventory of Safety Equipment. Progress maintenance as detailed in Monthly PMS. |  |  |
| 7 | Fire Hydrants | Test all hydrants for ease of operation. Lightly grease working parts. Leaking hydrants to be repaired. If required repaint hydrants RED |  |  |
|  |  | Hydrants caps to be in position, secured to hydrants with light chains and pressure relief holes clear. |  |  |
| 8 | Emergency Stops | All remote controls to be tested for correct functions in :- a) Stopping ventilation fans and pumps, and b) Closing cocks or valves to storage, settling or service tanks. QC valves. |  |  |
| 9 | Fixed Foam Fire Fighting System | Using water, operate system for a period of approximately 30 seconds, checking operation of all monitors and protable applicators. |  |  |
|  |  | Circulate foam through foam pump and back to foam tank to test pump operation. |  |  |
|  |  | Check all system valves are left in correct position. |  |  |
| 10 | Emergency Lighting | Accommodation and engine room emergency lighting to be tested to ensure correct functioning of all fittings and lamps. Arrange for repair or replacement of defective items. |  |  |

| 11 | Emergency Steering Gear | In addition to the routine checks and tests prescribed in paragraphs (a) and (b) SOLAS Chapter V Reg. 19-2, emergency steering gear drills should be held at least every 3 months. These drills shall include direct control from within the steering gear compartment, the communications procedure with the navigating bridge and, where applicable, the operation of alternate power supplies. |  |  |
| --- | --- | --- | --- | --- |
|  |  | The dates of the checks and tests prescribed in paragraphs (a) and (b) of SOLAS Chapter V Reg. 19-2, together with the dates of Emergency steering Drills, should be entered in the vessel’s Log Book. |  |  |
|  |  | Test means of communication (Batt. Power Tel.) between bridge, engine room and emergency steering (SOLASV 12.f) |  |  |

- HALF YEARLY ROUTINE :

| No. | Item | Checks & Maintenance | Done On | Next Due |
| --- | --- | --- | --- | --- |
| 1 | Life rafts | Check painters are in good condition and firmly secured to ship using weak link arrangement. |  |  |
|  |  | Inspect lashings and renew if necessary. |  |  |
|  |  | Hydrostatic releases to be manually operated and working parts lightly greased, in accordance manufacturer’s instructions. |  |  |
|  |  | Check life raft containers are marked with ship’s name and Port of Registry. |  |  |
|  |  | Check date of survey and when due survey. |  |  |
|  |  | Check condition and location of light and boarding line for forward raft. |  |  |
| 2 | Lifeboat / Rescue Hulls | Inspect hulls for damage and chafing. Repair or report as necessary. |  |  |
|  |  | Check that lifeboats are marked in accordance with SOLAS 1, Ch. 3, 41.9 |  |  |
|  |  | Check position and condition of Retro-reflective tape M Notice 1444. |  |  |
| 3 | Lifeboat / Rescue Boat and Life raft Embarkation Ladders | Ladders to be spread and rope work, chocks and step inspected. Repair renew as necessary. Check securing arrangements. |  |  |
| 4 | Bridge Pyrotechnics, MOB Signals | Inspect ship’s distress signals and line throwing equipment for conditions and correct stowage. |  |  |
|  |  | Check that Man Overboard Signals are secured by lanyards to quick release lifebuoys on bridge wings. |  |  |
|  |  | Check pyrotechnics against the Pyrotechnics Record Sheet for details of renewal. |  |  |
| 5 | Navigation Shapes, Lights and Sound Signals | Check condition of Navigation Shapes. Repaint, repair or replace as necessary. |  |  |
|  |  | Check sufficient spare lamps and slides for Navigational Lights are carried. |  |  |
|  |  | Check operation of:- a)Fog gong and striker. b)Portable for horn. c)Bells and striker. |  |  |
|  |  | Check condition of signal flags and halyards. |  |  |
| 6 | Lifebuoys | Inspect all lifebuoys for damage or deterioration and replace where req. |  |  |
|  |  | Re-mark with ship’s name and Port or Registry where necessary. |  |  |
|  |  | Check self-igniting lights and buoyant lines are in good condition. Renew where necessary. |  |  |
|  |  | Check that Man Overboard lifebuoys are free to operate. SOLAS III7.1.3). |  |  |
|  |  | Check retro-reflective tape is in good condition and positioned in accordance With ‘M’ Notice 1444. |  |  |
| 7 | Fire Blankets | Check fire blankets are in good condition and free from oil, grease or paint. |  |  |

| 8 | # Spare Fire Fighting and Safety Equipment | Check containers are secured clearly marked and in right location. Check spare equipment; indent for items required to maintain the recommended stock to be carried. |  |  |
| --- | --- | --- | --- | --- |
| 9 | Portable Gas Fire Extinguishers | Check discharge hoses for signs of perishing. Replace if necessary. |  |  |
|  |  | Ensure that discharge horns are fitted correctly. |  |  |
|  |  | Weigh each cylinder and if the weight of contents is less than 90% of the indicated weight replace with a fully charged spare. |  |  |
| 10 | Watertight, Gastight & Fire Doors | Watertight Doors: lubricate hinges and dogs, check - sealing gasket for damage or contamination by paint. |  |  |
|  |  | Gastight Doors: lubricate hinges and dogs, check - sealing gasket for damage or contamination by paint. |  |  |
|  |  | Fire Doors – lubricate hinges, check correct oeration of doors. Door to be labeled “FIRE DOOR”. |  |  |
|  |  | Damaged Doors to be listed for repair. |  |  |
| 11 | Fixed Gas Fire Smothering System | Inspect cylinders, pipe work, wires and pulls for correct connections. |  |  |
|  |  | Verify cylinders are securely clamped in position and correctly aligned. |  |  |
|  |  | Check gauge levels of cylinders. |  |  |
|  |  | Blow through pipelines if appropriate air connection is fitted. |  |  |
|  |  | Test operation of alarms and shut – offs. |  |  |
| 12 | Control Valves for Fixed Gas Smothering System | Check that all control valves or cocks are clearly marked to indicate the compartments to which the pipe are led. |  |  |

- YEARLY ROUTINE :

| No. | Item | Checks & Maintenance | Done On | Next Due |
| --- | --- | --- | --- | --- |
| 1 | Life rafts | Life rafts and their hydrostatic releases, when applicable are required to be landed annually for servicing. |  |  |
|  |  | Details of rafts are to be entered as below on the Maintenance & Record Card. Serial No.. Capacity (persons). Position; Stbd; Port. Date and Place of Last Service. Next Service Due. H R U Last Service. H R U Next Service Due. |  |  |
| 2 | Pyrotechnics | Complete the Pyrotechnics Record Sheet with details of all distress rockets, distress flares, man overboard signals, smoke floats and line throwing apparatus |  |  |
|  |  | Initial the Maintenance & Record Card annually to verify that all pyrotechnics are in good condition, correct number held and are in date. |  |  |
| 3 | Fire Extinguishers | All portable fire extinguishers are to be marked with a ship’s serial number. The number is also to be marked alongside the stowage position of each extinguisher. |  |  |
|  |  | Complete the Fire Extinguishers Record Sheet with the details required. |  |  |
|  |  | Fire Extinguishers are to be pressure tested at intervals not exceeding three (3) years, with the exception of Halon extinguisher. |  |  |
|  |  | Initial the Maintenance & Record Card annually to verify that all extinguishers are in place and in date for pressure testing. |  |  |
| 4 | Lifeboat/ Rescue Boat Falls | Complete details as below on the Maintenance & Record Card. |  |  |
|  |  | Falls are to be “end-for-ended” every 2.5 years and renewed every 5 years. |  |  |
|  |  | Initial the Maintenance & Record Card annually to verify that all falls are in good condition and in date either for renewing or “end-for-ending”. |  |  |
|  |  | FALLS | No.1 BOAT | NO. 2 BOAT |
|  |  | Size Diam. mm. |  |  |
|  |  | Safe Working Load |  |  |
|  |  | Date Falls Renewed |  |  |

Remarks :

SIGN OFF :             Chief  Officer                                                              2nd Engineer

Chief Engineer                                                 Master